SUMPTER TOWNSHIP

23480 Sumpter Rd. Belleville, MI 48111

(734) 256-4420

DETACHED ACCESSORY BUILDINGS GUIDELINES

YOU ARE REQUIRED TO HAVE THE FOLLOWING ITEMS BEFORE A

BUILDING PERMIT IS ISSUED FOR AN ACCESSORY BUILDING

(Garage/Pole Barn/Etc.)

- Application- with plot plan showing the setbacks

- $75.00 plan review fee

- The building location must be staked out with 4-foot stakes. Township lines (front & one side) must also be staked.

- Architectural plans which adhere to the requirements of the 2015 Michigan Residential Code (MRC)

- Tax ID # from tax bill

- Copy of ownership records (ex: Warranty Deed, Land Contract, Property Transfer Affidavit, etc.)

- Two sets of blueprint or drawings of the proposed structure. Please include cross sections, elevations and (truss design from manufacturer). One set will be returned after approval from the Building Inspector.

- Homeowner Affidavit- Unless proposed structure is being built by someone other than the owner, a copy of the builder’s state license.

A MINIMUM OF 3 INSPECTIONS MUST BE CALLED FOR:

- Footings or applicable foundation/post holes.

- Rough Inspection- before covering interior structural members, included pre-slab inspection.

- Final Inspection- concrete floor, all painting or staining (if applicable) and final grade (seeded).

NOTICE: The water table affects most footing locations in Sumpter Township and the surrounding area. It is recommended by this office that the bottom of the footing by no less than 1 inch above the water table. It is therefore recommended that a posthole be dug at the time of staking to a depth of 4 feet. Before excavation, the hole will be inspected by the excavator. Example: If water in the posthole is at 30 inches below the existing grade, the bottom of the footing shall be no more than 29 inches below the existing grade.

Section 5.3. - Accessory buildings, structures and uses.

All accessory buildings, except as otherwise permitted in this ordinance, shall be subject to the following regulations:

- Detached accessory buildings (residential use). Detached accessory buildings which are accessory to single-family residential uses in all residential districts, including the AG, RF, R, RM and RMH districts, with the exception of farm buildings and structures associated with a legally existing farm operation protected under the Right to Farm Act, shall conform to the following requirements:

- Relation to principal building. Accessory buildings, structures, and uses are permitted only in connection with, incidental to, and on the same lot or parcel with a principal building that is occupied by a use permitted in the particular zoning district. In the AG- Agricultural district, an accessory building or structure may be permitted on a separate lot or parcel in conjunction with activity of a permitted use on another lot or parcel under the same ownership.

- Permit required and exemptions. Before a building permit can be issued, a certificate of zoning compliance shall be sought and will be issued if determined by the zoning administrator that all ordinance provisions have been satisfied. The following accessory buildings are exempt from building permit application, however, the all other applicable regulations and standards, as set forth in this section, shall be met:

- One shed of up to two hundred (200) square feet in area, no more than fourteen (14) feet tall and located within the rear yard.

- School bus shelters.

- Temporary buildings for the incidental sale of crops or products grown on the premises.

- Maximum number. No more than two (2) detached accessory buildings shall be permitted on any lot or parcel. This standard does not apply to properties located within the agricultural (AG) or rural small farm (RF) district; however, the total combined gross floor area for detached buildings in all districts shall not exceed that permitted in subsection (4) below.

- Maximum size. The maximum allowable size for the total of all detached buildings on an individual site shall be determined according to the following table:

| Parcel Size | Maximum Combined Gross Floor Area |
| --- | --- |
| 0.5 acres or less | 1,200 sq. ft. |
| 0.51 acres to 1 acre | 1,800 sq. ft. |
| 1.01 acres to 3 acres | 2,400 sq. ft. |
| 3.01 acres to 5 acres | 3,000 sq. ft. |
| 5.01 acres to 7 acres | 3,800 sq. ft. |
| 7.01 acres or larger | 6,000 sq. ft. |

- On lots 3 acres or less in size, the gross floor area of an individual detached accessory building shall not exceed that of an existing principal building on site.

- The maximum detached accessory building size for properties located within the RMH, manufactured housing park district, shall be two hundred (200) square feet.

- Required setbacks and maximum height. Except as noted below, detached accessory buildings shall not be constructed closer to the front property line than any portion of the principal building and shall comply with the following setback and height requirements.

| Building Size | Maximum Building Height | Minimum Side Setback | Minimum Rear Setback | Minimum Setback Between Buildings |
| --- | --- | --- | --- | --- |
| 800 sq. ft. or less | 14 ft. | 10 ft. | 10 ft. | 10 ft. |
| 801 sq. ft. to 1,200 sq. ft. | 18 ft. | 10 ft. | 10 ft. | 10 ft. |
| 1,201 sq. ft. to 1,800 sq. ft. | 18 ft. | 15 ft. | 15 ft. | 10 ft. |
| 1,801 sq. ft. to 2,500 sq. ft. | 18 ft. | 15 ft. | 15 ft. | 15 ft. |
| 2,501 sq. ft. to 3,300 sq. ft. | 25 ft. | 20 ft. | 20 ft. | 15 ft. |
| 3,301 sq. ft. or larger | 30 ft. | 30 ft. | 30 ft. | 25 ft. |

- Detached accessory buildings may be permitted closer to the front property line than the principal building where all of the following conditions are met:

- The property is located within the AG or RF district.

- The property is over three (3) acres in size.

- The accessory building is located at least seventy-five (75) feet from the front property line.

- The design and architecture of the accessory building shall be compatible in      appearance with adjacent development.

- If located on a corner lot or parcel, the detached accessory building may be located in the non-addressed front yard providing the front yard setback is comparable to the principal building on the adjacent lot or parcel. If located adjacent to a vacant lot or parcel, the accessory building shall meet the front yard setback of the particular zoning district for the non-addressed front yard.

- Architectural compatibility. All accessory buildings shall be architecturally compatible with the principal building (i.e., building materials, roof pitch, height, etc.). A determination of architectural compatibility can be referred to the planning commission at the sole discretion of the zoning administrator.

- Habitable space. Habitable space is allowed within a detached accessory building subject to the special land use criteria and procedures of section 3.3 and the following:

- No more than fifty (50) percent of the total floor area (combined first and second floors where applicable) may be occupied as habitable space.

- The space may only be occupied by an immediate family member (i.e., father, mother, daughter, son, grandparent, and grandchild).

- Under no circumstance shall the space be rented or cause to be occupied by someone other than an immediate family member.

- All such detached accessory buildings shall be located no less than twenty (20) feet from a rear or side property line and thirty (30) feet from another building.

- Farm animals/by-products. Structures housing farm animals or animal-by-products shall comply with the setback standards of section 6.40, unless exempted by the Michigan Right to Farm Act.

(Zoning Ord. of 2-10-09(6); Zoning Ord. of 9-9-14; Ord. of 1-12-16(1); Ord. of 1-23-2018(1), Art. 3, § 1; Ord. of 3-12-19)

SITE PREPARATION FOR DETACHED BUILDINGS

Standards for Detached Garages (daily use of cars & trucks), along with pole frame constructed buildings used for storage are listed below:

- Both buildings require a minimum finish floor height of not less than one (1) foot above the existing grade, where the building is being placed on site.

- All organic material must be removed from the area that the building covers. (Insert compaction material for under concrete slabs, for motorized equipment, and household storage). Floors for animals do not require compaction material.

- The color of both buildings shall be compatible with the dwelling (EXCEPTION) primary use relates to agricultural activity.

- Footings for detached garages shall be continuous and no less than eight (8) inches wide along with forty-two (42) inches below the new established grade.

- Pole frame constructed buildings used for storage or agricultural activities require a twelve (12) inch wide, by six (6) inch deep pad for each vertical structural member. (1 bag of R/M concrete). As above, forty-two (42) inches below the new established grade.

- Doors facing the main road shall not exceed more than eight (8) feed in height for both of the above buildings, (EXCEPTION) Agricultural activities and recreational vehicles higher than eight (8) feet.

- The raising of fowl or their by-products for commercial purposes shall be conducted wholly within an enclosed building. All waste parts or offal must be IMNÆDIATELY disposed of, and no outdoor storage of offal shall be permitted.

- Any pen, corral or structure where barnyard animals are maintained as a feedlot shall be located a minimum of three thousand (3000) feet to the nearest residential district. A feedlot shall not be located closer than one hundred fifty (150) feet from a residence.